Part 1: Capacity and Overall Design

Total Capacity: 150 liters (39.6 gallons). This capacity is specifically engineered for craft distilleries and serious enthusiasts, balancing production volume with operational flexibility.

Working Capacity: 100-120 liters (26.5-31.7 gallons) per distillation cycle, accounting for 20-30% headspace to prevent boil-over and ensure safe operation during vigorous fermentation or high-temperature distillation.

Construction Material:

Main Body: Fabricated from 2.5mm-thick food-grade 304 stainless steel (with 316L option for acidic mashes), complemented by copper cladding on critical surfaces. The hybrid construction combines stainless steel’s durability with copper’s catalytic properties.

Key Components:

Column Internals: Constructed from 2mm-thick oxygen-free copper (C10100) for optimal sulfur binding and esterification.

Condenser Tubes: Deoxidized high-phosphorus copper (C12200) for enhanced heat transfer and corrosion resistance.

Flavor Basket: Woven copper mesh (0.8mm wire diameter) with quick-release mechanism for botanical swapping during operation.

Design Style:

Modularity: Tri-clamp (ASA 1.5") connections throughout, allowing reconfiguration within 15 minutes (e.g., switching from pot still to reflux mode).

Finish Options: #4 Brushed finish (standard), #8 Mirror polish (premium), or powder-coated exterior (custom colors available).

Ergonomics: Elevated kettle design with 45° angled column connection for natural vapor flow. Integrated lifting lugs for crane assistance.

Part 2: Heating System

Heating Methods:

Electric Heating:

Elements: Incoloy-sheathed tubular elements (3-6 pieces), radially arranged for even heat distribution.

Configuration: Bottom-mounted with optional side-mounted "boosters" for faster heat-up times (≈45 minutes to boiling from 20°C).

Steam Heating:

Jacket Options: Half-jacket (50% coverage) or full-jacket (90% coverage) with 150PSI ASME certification.

Steam Requirement: 45-60 kg/h at 100-125PSI for continuous operation.

Direct Fire:

Optional Leg Kit: Raises kettle 400mm for propane burner installation (e.g., 200,000 BTU banjo burner).

Power Customization:

Voltage: 208V/240V/480V compatible with dual-voltage wiring.

Phase Configuration: Single-phase (up to 24kW) or three-phase (up to 72kW) with silicon-controlled rectifier (SCR) power regulation.

Step-less Control: 0-100% power adjustment via PWM signals to SSR switches.

Part 3: Power Supply and Control System

Electrical Specifications:

Main Disconnect: 100A NEMA 4X rated enclosure with lockout/tagout capability.

Wiring: 4-wire system (L1/L2/L3 + G) with 6mm² THHN copper wiring throughout.

Control System:

Hardware: 7" HMI touchscreen (IP65 rated) with Raspberry Pi CM4 or Siemens S7-1200 PLC backend.

Software Features:

Recipe Management: Store 50+ distillation programs (e.g., whiskey stripping run, vodka spirit run).

Adaptive PID: Auto-tunes heating parameters based on wash composition and ambient conditions.

Remote Monitoring: OPC UA/Modbus TCP protocol for integration with SCADA systems.

Safety Systems:

Category 3 PLe Safety Circuit: Includes emergency stop, flow sensors for cooling water, and pressure transducers on column.

Ground Fault Protection: 30mA sensitivity on all heating circuits.

Part 4: Column and Condenser

Column Specifications:

Plate Design: 4-5 bubble cap plates with 100mm plate spacing. Caps manufactured from CNC-machined copper with 2mm vapor slots.

Packing Options:

Standard: Copper mesh rolls (99.9% purity)

Premium: Structured packing (BX500 style) or ceramic saddles

Height Customization: 900mm (3-plate) to 1500mm (5-plate) with intermediate extensions available.

Monitoring: 2× borosilicate sight glasses (25mm thick) with LED backlighting option.

Condenser System:

Design: Counterflow shell-and-tube with 19× tubes (12mm OD, 0.8mm wall) providing 1.8m² heat exchange area.

Performance: Cools 30L/h of vapor to 20°C with 15°C coolant input at 20L/min flow rate.

Connections: 1" NPT ports for coolant with integrated temperature sensors.

Part 5: Other Features and Accessories

Specialized Components:

Dephlegmator: Secondary condenser mounted atop column for precise reflux control (0-100% adjustable).

Product Divider: Three-way ball valve with Teflon seats for heads/hearts/tails separation without interrupting flow.

CIP System: Rotary spray ball (316SS) with 1.5" TC connection for automated cleaning cycles.

Instrumentation:

Temperature: PT100 sensors (4×) with ±0.1°C accuracy at all critical points.

Pressure: Digital manometer (0-1 bar range) with high-limit cutoff at 0.8 bar.

Alcohol Concentration: In-line refractometer (0-100% ABV range) with temperature compensation.

Certifications:

ASME BPVC Section VIII Div. 1 pressure vessel certification

CE/PED 2014/68/EU compliance

NSF/ANSI 58 certification on food-contact surfaces